METALLURGY OF ZINC

Exam-Pattern Answers

# Q.1  Extraction of Zinc — Vertical Retort Process

The ore containing zinc blende is concentrated and further reduced by the vertical retort process to obtain molten zinc.  [3 + 1 + 1]

a. Write all the chemical reactions taking place in the extraction of zinc from its ore.

Concentration: The ore (zinc blende) is concentrated by froth floatation.

Roasting: The concentrated ore is roasted in excess air, converting sulphide to oxide.

2ZnS + 3O2  →  2ZnO + 2SO2↑

Reduction (vertical retort): ZnO mixed with coke is heated to about 1673 K in fireclay retorts. Zinc oxide is reduced to zinc, which distils off as vapour along with CO gas.

ZnO + C  →  Zn (vapour) + CO↑

Condensation: The vapours pass into a condenser where zinc liquefies and is cast into ingots (spelter, ~97–98% pure); CO escapes and is burnt as fuel.

b. How are metal impurities like Cd, Fe, Pb removed from zinc spelter?

By two successive physical methods:

•  Liquation: Impure zinc is heated just above its melting point (≈419°C) on a sloping hearth; zinc melts and flows away, while high-melting Fe remains behind as solid residue.

•  Distillation: The liquated zinc is fractionally distilled — zinc distils over and is collected, while more volatile Cd comes off first and less volatile Pb remains as residue.

c. Electrolytic purification: cathode material; cathode and anode reactions.

Cathode: thin sheet of pure aluminium.  Anode: block of impure zinc.  Electrolyte: acidified ZnSO4 solution.

Anode:    Zn  →  Zn2+ + 2e-

Cathode:  Zn2+ + 2e-  →  Zn

[Figure: Electrolytic cell — impure zinc anode and pure aluminium cathode in acidified ZnSO4 solution, connected to a DC source; pure zinc deposits on the cathode, anode mud settles below.]

# Q.2  Properties of Zinc after Extraction

Fine zinc metal is obtained after a series of metallurgical processes.  [1 + 2 + 1 + 1]

a. What happens when the obtained fine zinc is left in the atmosphere?

It loses its shine and gets coated with a dull grey layer of basic zinc carbonate, ZnCO3·Zn(OH)2, formed by reaction with moisture, O2 and CO2 in air. This layer is adherent and protects the metal from further corrosion.

b. Reaction of zinc with (i) hot concentrated H2SO4 and (ii) very dilute HNO3.

(i) Zinc reduces hot concentrated H2SO4, liberating SO2 gas:

Zn + 2H2SO4 (conc., hot)  →  ZnSO4 + SO2↑ + 2H2O

(ii) With very dilute HNO3, zinc reduces it right down to ammonium nitrate instead of liberating H2:

4Zn + 10HNO3 (very dilute)  →  4Zn(NO3)2 + NH4NO3 + 3H2O

c. Does zinc displace copper from copper sulphate solution? Justify.

Yes. Zinc lies above copper in the electrochemical series, i.e., it is more reactive, so it displaces copper from CuSO4 solution — the blue colour fades as reddish copper deposits.

Zn + CuSO4  →  ZnSO4 + Cu↓

d. Reaction of zinc with potassium hydroxide.

Zinc is amphoteric and dissolves in hot concentrated KOH, liberating H2 and forming soluble potassium zincate:

Zn + 2KOH (hot, conc.)  →  K2ZnO2 (potassium zincate) + H2↑

# Q.3  Reaction of Zinc with Dilute Sulphuric Acid

The finally obtained pure zinc reacts with dilute sulphuric acid to give a white crystalline solid.  [1 + 2 + 2]

a. Name the compound and give its molecular formula.

The white crystalline solid is Zinc Sulphate heptahydrate, commonly called 'white vitriol'.

Molecular formula:  ZnSO4.7H2O

Zn + H2SO4 (dilute)  →  ZnSO4 + H2↑

b. One method of preparation; equation with barium sulphide.

ZnSO4 is prepared by treating zinc/zinc oxide/zinc carbonate with dilute H2SO4, then crystallising. By double decomposition with barium sulphide:

ZnSO4 + BaS  →  ZnS↓ + BaSO4↓

c. Action of heat on the white crystalline solid; reaction with excess alkali.

On heating, ZnSO4·7H2O first loses its water of crystallisation to give white anhydrous ZnSO4; on further strong heating, it decomposes to zinc oxide, releasing sulphur trioxide:

ZnSO4.7H2O  --(heat)-->  ZnSO4 (anhydrous) + 7H2O

ZnSO4  --(strongly heated)-->  ZnO (white, turns yellow when hot) + SO3↑

With excess alkali (NaOH), the initial white precipitate of Zn(OH)2 dissolves (zinc hydroxide is amphoteric) to form soluble sodium zincate:

ZnSO4 + 2NaOH  →  Zn(OH)2↓ + Na2SO4

Zn(OH)2 + 2NaOH (excess)  →  Na2ZnO2 (sodium zincate) + 2H2O

# Q.4  A Non-Typical Transition Metal — Displaces Ag from AgNO3

The metal, when placed in AgNO3 solution, displaces Ag; it also becomes dull in moist air.  [1 + 3 + 1]

a. Identify the metal.

The metal is Zinc (Zn). Being more reactive than silver, it displaces silver from silver nitrate solution:

Zn + 2AgNO3  →  Zn(NO3)2 + 2Ag↓

Zinc is termed a 'non-typical' transition metal because, although it is placed in the d-block, both the metal (3d10 4s2) and its common ion Zn2+ (3d10) have a completely filled d-subshell, so it lacks the characteristic properties of typical transition elements (variable oxidation states, coloured ions, catalytic activity, complex formation are largely absent/limited).

b. Reduction step during extraction from its sulphide ore, with a labelled diagram.

After roasting ZnS to ZnO, the oxide is mixed with coke and heated to about 1673 K in a fireclay retort. Carbon reduces ZnO to zinc, which distils off as vapour and is condensed in a receiver.

ZnO + C  →  Zn (vapour) + CO↑

[Figure: Vertical retort charged from the top with roasted ore + coke, heated externally; zinc vapour passes into a side condenser and liquefies; CO gas escapes and is burnt.]

c. Convert the metal into its vitriol.

Zinc is dissolved in dilute sulphuric acid, and the resulting solution is evaporated and crystallised to obtain white vitriol:

Zn + H2SO4 (dilute)  →  ZnSO4 + H2↑

On crystallisation:  ZnSO4  +  7H2O  →  ZnSO4.7H2O (white vitriol)

# Q.5  Metal 'M' — [Ar]3d10 4s2, Group IIB, 'Jasta'

Metal M has electronic configuration [Ar]3d10 4s2, belongs to group IIB, and is commonly called 'jasta' (yashad).  [1 + 1 + 2 + 1]

a. Identify metal M and write its main ore.

M = Zinc (Zn). Its principal ore is Zinc Blende (ZnS); other ores include Calamine (ZnCO3) and Zincite (ZnO).

b. Why is metal M not considered a typical transition metal?

In Zn, the 3d-subshell is completely filled with 10 electrons, both in the free metal (3d10 4s2) and in its common ion Zn2+ (3d10). Since transition character arises from having a partially filled d-subshell (in the atom or in at least one common oxidation state), zinc does not show the typical transition-metal properties — it exhibits only one oxidation state (+2), forms colourless (white) compounds, and shows little catalytic or complex-forming behaviour. Hence it is regarded as a non-typical (or non-true) transition element.

c. Vertical retort — diagram and reduction reaction.

[Figure: Vertical retort — a tall, cylindrical fireclay retort charged from the top with briquettes of roasted ore (ZnO) mixed with coke; the retort is heated externally to about 1673 K; zinc vapour formed passes through a side condenser and is collected as molten zinc, while CO gas escapes and is burnt off at the top.]

ZnO + C  --(≈1673 K)-->  Zn (vapour) + CO↑

d. What happens when metal M is exposed to air for a long time?

On prolonged exposure to moist air, zinc reacts with moisture, carbon dioxide and oxygen to form a dull grey-white protective coating of basic zinc carbonate, which adheres to the surface and prevents further corrosion of the metal beneath.

2Zn + O2 + H2O + CO2  →  ZnCO3.Zn(OH)2 (basic zinc carbonate)